Baseline Summary B1294 (2966 to ORLA)

| Processed: | Friday, Apr 02, 2004 06:52:59AM |
| --- | --- |
| Solution type: | L1 fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 3760.619m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 12.2 |
| Reference variance: | 2.574 |
| RMS: | 0.005m |
| Horizontal Precision 1-sigma (scaled): | 0.001m |
| Vertical Precision 1-sigma (scaled): | 0.002m |

| Start time (GPS Time): | 04/03/19, 15:54:45.000 | 1262, 489285.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/19, 16:45:15.000 | 1262, 492315.000 |
| Occupation time: | 00:50:30.000 |  |

Baseline Components (Mark to Mark)

| From: | 2966 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 735774.141m | Latitude | 39°47'25.30909"N | Latitude | 39°47'25.30909"N |
| Easting | 1980652.599m | Longitude | 122°13'33.16735"W | Longitude | 122°13'33.16735"W |
| Elevation | 91.757m | Height | 63.159m | Height | 63.159m |

| To: | ORLA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 733340.413m | Latitude | 39°46'06.61214"N | Latitude | 39°46'06.61213"N |
| Easting | 1983519.399m | Longitude | 122°11'32.46022"W | Longitude | 122°11'32.46022"W |
| Elevation | 82.215m | Height | 53.719m | Height | 53.719m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -2433.728m | NS Fwd Azimuth | 130°11'12" |  X | 1606.513m |
|  Easting | 2866.800m | Ell. Distance | 3760.573m |  Y | -2838.908m |
|  Elevation | -9.542m |  Height | -9.440m |  Z | -1871.357m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.026 seconds |  X | 0.001m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.001m |
|  Elevation | 0.001m |  Height | 0.001m |  Z | 0.001m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 4.957e-7m2 |  |  |
| Y | 4.660e-7m2 | 1.247e-6m2 |  |
| Z | -4.428e-7m2 | -7.653e-7m2 | 9.550e-7m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | 2966 | ORLA |
| Data file: |  | 29660791.DAT | ORLA0791.DAT |
| Receiver Type: |  | 4000SSi | 4000SSi |
| Receiver Serial Number: |  | 6782 | 14631 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 2.063m | 2.063m |
|  | APC | 2.063m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |