Baseline Summary B1340 (P30W to ARTO)

| Processed: | Friday, Apr 02, 2004 06:52:59AM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 5575.134m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 16.6 |
| Reference variance: | 0.365 |
| RMS: | 0.007m |
| Horizontal Precision 1-sigma (scaled): | 0.001m |
| Vertical Precision 1-sigma (scaled): | 0.001m |

| Start time (GPS Time): | 04/03/19, 21:43:45.000 | 1262, 510225.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/19, 22:30:15.000 | 1262, 513015.000 |
| Occupation time: | 00:46:30.000 |  |

Baseline Components (Mark to Mark)

| From: | P30W |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 720483.265m | Latitude | 39°39'09.93757"N | Latitude | 39°39'09.93757"N |
| Easting | 1987023.140m | Longitude | 122°09'04.34611"W | Longitude | 122°09'04.34611"W |
| Elevation | 55.317m | Height | 26.817m | Height | 26.817m |

| To: | ARTO |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 717336.562m | Latitude | 39°37'27.61183"N | Latitude | 39°37'27.61182"N |
| Easting | 1982421.218m | Longitude | 122°12'17.08702"W | Longitude | 122°12'17.08702"W |
| Elevation | 59.701m | Height | 30.968m | Height | 30.968m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -3146.702m | NS Fwd Azimuth | 235°32'29" |  X | -4963.860m |
|  Easting | -4601.922m | Ell. Distance | 5575.108m |  Y | 740.864m |
|  Elevation | 4.384m |  Height | 4.150m |  Z | -2427.619m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.045 seconds |  X | 0.001m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.001m |
|  Elevation | 0.002m |  Height | 0.002m |  Z | 0.002m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 8.465e-7m2 |  |  |
| Y | 7.509e-7m2 | 1.644e-6m2 |  |
| Z | -5.585e-7m2 | -8.036e-7m2 | 3.448e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | P30W | ARTO |
| Data file: |  | P30W0794.DAT | ARTO0794.DAT |
| Receiver Type: |  | 4700 | 4000SSi |
| Receiver Serial Number: |  | 220203616 | 7618 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Bottom of antenna mount | Antenna Phase Center |
| Antenna height | Measured | 2.000m | 2.063m |
|  | APC | 2.063m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |