Baseline Summary B1421 (L191 to WILS)

| Processed: | Friday, Apr 02, 2004 06:52:59AM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 6267.312m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 12.2 |
| Reference variance: | 0.624 |
| RMS: | 0.008m |
| Horizontal Precision 1-sigma (scaled): | 0.004m |
| Vertical Precision 1-sigma (scaled): | 0.006m |

| Start time (GPS Time): | 04/03/19, 21:45:30.000 | 1262, 510330.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/19, 22:30:15.000 | 1262, 513015.000 |
| Occupation time: | 00:44:45.000 |  |

Baseline Components (Mark to Mark)

| From: | L191 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 712628.937m | Latitude | 39°34'55.37527"N | Latitude | 39°34'55.37526"N |
| Easting | 1989492.170m | Longitude | 122°07'20.33333"W | Longitude | 122°07'20.33333"W |
| Elevation | 42.571m | Height | 14.016m | Height | 14.016m |

| To: | WILS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 711398.220m | Latitude | 39°34'15.12120"N | Latitude | 39°34'15.12120"N |
| Easting | 1983347.209m | Longitude | 122°11'37.72887"W | Longitude | 122°11'37.72887"W |
| Elevation | 44.566m | Height | 15.752m | Height | 15.752m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -1230.717m | NS Fwd Azimuth | 258°35'52" |  X | -5622.363m |
|  Easting | -6144.961m | Ell. Distance | 6267.298m |  Y | 2598.984m |
|  Elevation | 1.995m |  Height | 1.736m |  Z | -955.785m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.005m |  NS Fwd Azimuth | 0.150 seconds |  X | 0.002m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.002m |
|  Elevation | 0.007m |  Height | 0.007m |  Z | 0.008m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 3.310e-6m2 |  |  |
| Y | 3.911e-6m2 | 5.836e-6m2 |  |
| Z | -1.239e-5m2 | -1.522e-5m2 | 6.157e-5m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | L191 | WILS |
| Data file: |  | L1910794.DAT | WILS0794.DAT |
| Receiver Type: |  | 4000SSi | 4000SSE |
| Receiver Serial Number: |  | 6782 | 1547 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 2.063m | 2.063m |
|  | APC | 2.063m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |