Baseline Summary B1449 (S106 to W215)

| Processed: | Friday, Apr 02, 2004 05:53:55PM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 8444.255m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 14.5 |
| Reference variance: | 1.526 |
| RMS: | 0.013m |
| Horizontal Precision 1-sigma (scaled): | 0.003m |
| Vertical Precision 1-sigma (scaled): | 0.008m |

| Start time (GPS Time): | 04/03/19, 19:56:15.000 | 1262, 503775.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/19, 20:46:15.000 | 1262, 506775.000 |
| Occupation time: | 00:50:00.000 |  |

Baseline Components (Mark to Mark)

| From: | S106 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 728055.228m | Latitude | 39°43'11.19048"N | Latitude | 39°43'11.19047"N |
| Easting | 1952886.737m | Longitude | 122°32'58.17924"W | Longitude | 122°32'58.17924"W |
| Elevation | 279.889m | Height | 250.991m | Height | 250.991m |

| To: | W215 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 736493.297m | Latitude | 39°47'44.84263"N | Latitude | 39°47'44.84262"N |
| Easting | 1953190.895m | Longitude | 122°32'47.53935"W | Longitude | 122°32'47.53935"W |
| Elevation | 210.886m | Height | 181.956m | Height | 181.956m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 8438.069m | NS Fwd Azimuth | 1°43'05" |  X | 3146.135m |
|  Easting | 304.158m | Ell. Distance | 8443.686m |  Y | 4458.544m |
|  Elevation | -69.003m |  Height | -69.036m |  Z | 6444.274m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.002m |  NS Fwd Azimuth | 0.044 seconds |  X | 0.002m |
|  Easting | 0.002m |  Ell.Distance | 0.002m |  Y | 0.005m |
|  Elevation | 0.006m |  Height | 0.006m |  Z | 0.003m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 5.483e-6m2 |  |  |
| Y | 1.025e-5m2 | 2.933e-5m2 |  |
| Z | -6.853e-6m2 | -1.478e-5m2 | 1.145e-5m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | S106 | W215 |
| Data file: |  | S1060793.DAT | W2150793.rnx |
| Receiver Type: |  | 4700 | Z-12 |
| Receiver Serial Number: |  | 220202606 | 788 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Ashtech Geodetic III L1/L2 |
| Measured To: |  | Bottom of antenna mount | Bottom of antenna mount |
| Antenna height | Measured | 2.000m | 1.890m |
|  | APC | 2.063m | 1.962m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |