Baseline Summary B1495 (P30W to EXT1)

| Processed: | Friday, Apr 02, 2004 06:52:59AM |
| --- | --- |
| Solution type: | L1 fixed |
| Solution acceptability: | High reference variance |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 4923.587m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 105.0 |
| Reference variance: | 11.611 |
| RMS: | 0.011m |
| Horizontal Precision 1-sigma (scaled): | 0.003m |
| Vertical Precision 1-sigma (scaled): | 0.007m |

| Start time (GPS Time): | 04/03/30, 20:29:00.000 | 1264, 246540.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/30, 21:30:15.000 | 1264, 250215.000 |
| Occupation time: | 01:01:15.000 |  |

Baseline Components (Mark to Mark)

| From: | P30W |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 720483.265m | Latitude | 39°39'09.93757"N | Latitude | 39°39'09.93757"N |
| Easting | 1987023.140m | Longitude | 122°09'04.34611"W | Longitude | 122°09'04.34611"W |
| Elevation | 55.317m | Height | 26.817m | Height | 26.817m |

| To: | EXT1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 717916.663m | Latitude | 39°37'46.90410"N | Latitude | 39°37'46.90409"N |
| Easting | 1991224.589m | Longitude | 122°06'07.98517"W | Longitude | 122°06'07.98517"W |
| Elevation | 46.703m | Height | 18.323m | Height | 18.323m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -2566.601m | NS Fwd Azimuth | 121°19'29" |  X | 2695.926m |
|  Easting | 4201.448m | Ell. Distance | 4923.562m |  Y | -3614.351m |
|  Elevation | -8.614m |  Height | -8.494m |  Z | -1977.411m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.038 seconds |  X | 0.001m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.001m |
|  Elevation | 0.002m |  Height | 0.002m |  Z | 0.002m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 1.038e-6m2 |  |  |
| Y | 9.605e-7m2 | 1.963e-6m2 |  |
| Z | -9.240e-7m2 | -1.323e-6m2 | 2.646e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | P30W | EXT1 |
| Data file: |  | P3OW0903.DAT | EXT10903.DAT |
| Receiver Type: |  | 4000SSi | 4000SSi |
| Receiver Serial Number: |  | 14631 | 14632 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 1.952m | 1.952m |
|  | APC | 1.952m | 1.952m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |