Baseline Summary B1497 (HAMI to PETE)

| Processed: | Friday, Apr 02, 2004 06:52:59AM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 8887.998m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 75.0 |
| Reference variance: | 0.736 |
| RMS: | 0.010m |
| Horizontal Precision 1-sigma (scaled): | 0.001m |
| Vertical Precision 1-sigma (scaled): | 0.002m |

| Start time (GPS Time): | 04/03/30, 22:30:45.000 | 1264, 253845.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/30, 23:30:15.000 | 1264, 257415.000 |
| Occupation time: | 00:59:30.000 |  |

Baseline Components (Mark to Mark)

| From: | HAMI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 730643.309m | Latitude | 39°44'39.71968"N | Latitude | 39°44'39.71968"N |
| Easting | 1998236.837m | Longitude | 122°01'14.05672"W | Longitude | 122°01'14.05672"W |
| Elevation | 48.757m | Height | 20.829m | Height | 20.828m |

| To: | PETE |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 725257.778m | Latitude | 39°41'44.93984"N | Latitude | 39°41'44.93984"N |
| Easting | 1991166.607m | Longitude | 122°06'10.76544"W | Longitude | 122°06'10.76544"W |
| Elevation | 56.171m | Height | 27.908m | Height | 27.908m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -5385.531m | NS Fwd Azimuth | 232°41'25" |  X | -7820.377m |
|  Easting | -7070.231m | Ell. Distance | 8887.962m |  Y | 827.339m |
|  Elevation | 7.414m |  Height | 7.079m |  Z | -4141.706m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.029 seconds |  X | 0.001m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.002m |
|  Elevation | 0.002m |  Height | 0.002m |  Z | 0.002m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 1.669e-6m2 |  |  |
| Y | 1.353e-6m2 | 2.673e-6m2 |  |
| Z | -8.619e-7m2 | -1.330e-6m2 | 3.756e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | HAMI | PETE |
| Data file: |  | HAMI0904.DAT | PETE0904.DAT |
| Receiver Type: |  | 4000SSi | 4000SSi |
| Receiver Serial Number: |  | 14632 | 17633 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 1.952m | 2.063m |
|  | APC | 1.952m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |