Baseline Summary B1551 (OWEN to U107)

| Processed: | Friday, Apr 02, 2004 08:49:49AM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 9826.427m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 13.1 |
| Reference variance: | 0.527 |
| RMS: | 0.008m |
| Horizontal Precision 1-sigma (scaled): | 0.001m |
| Vertical Precision 1-sigma (scaled): | 0.001m |

| Start time (GPS Time): | 04/04/01, 16:11:15.000 | 1264, 403875.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/04/01, 17:15:00.000 | 1264, 407700.000 |
| Occupation time: | 01:03:45.000 |  |

Baseline Components (Mark to Mark)

| From: | OWEN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 699731.818m | Latitude | 39°27'56.44077"N | Latitude | 39°27'56.44077"N |
| Easting | 1978576.559m | Longitude | 122°14'56.27251"W | Longitude | 122°14'56.27251"W |
| Elevation | 41.503m | Height | 12.314m | Height | 12.314m |

| To: | U107 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 706989.969m | Latitude | 39°31'51.12039"N | Latitude | 39°31'51.12039"N |
| Easting | 1971953.653m | Longitude | 122°19'34.43679"W | Longitude | 122°19'34.43679"W |
| Elevation | 91.166m | Height | 61.998m | Height | 61.998m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 7258.151m | NS Fwd Azimuth | 317°27'48" |  X | -3180.026m |
|  Easting | -6622.906m | Ell. Distance | 9826.245m |  Y | 7409.544m |
|  Elevation | 49.663m |  Height | 49.684m |  Z | 5616.472m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.013 seconds |  X | 0.001m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.001m |
|  Elevation | 0.001m |  Height | 0.001m |  Z | 0.001m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 7.340e-7m2 |  |  |
| Y | 4.248e-7m2 | 1.152e-6m2 |  |
| Z | -4.272e-7m2 | -4.391e-7m2 | 8.761e-7m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | OWEN | U107 |
| Data file: |  | OWEN0921.DAT | U1070921.DAT |
| Receiver Type: |  | 4000SSE | 4000SSi |
| Receiver Serial Number: |  | 1547 | 14632 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Antenna Phase Center | Bottom of antenna mount |
| Antenna height | Measured | 1.952m | 2.000m |
|  | APC | 1.952m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |