Baseline Summary B1556 (V380 to Y380)

| Processed: | Sunday, May 02, 2004 07:39:20AM |
| --- | --- |
| Solution type: | L1 fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 4234.519m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 10.4 |
| Reference variance: | 3.706 |
| RMS: | 0.007m |
| Horizontal Precision 1-sigma (scaled): | 0.001m |
| Vertical Precision 1-sigma (scaled): | 0.003m |

| Start time (GPS Time): | 04/04/28, 15:21:15.000 | 1268, 314475.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/04/28, 16:30:00.000 | 1268, 318600.000 |
| Occupation time: | 01:08:45.000 |  |

Baseline Components (Mark to Mark)

| From: | V380 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 734898.543m | Latitude | 39°46'56.36838"N | Latitude | 39°46'56.36838"N |
| Easting | 1974727.716m | Longitude | 122°17'42.06800"W | Longitude | 122°17'42.06800"W |
| Elevation | 113.805m | Height | 84.989m | Height | 84.989m |

| To: | Y380 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 732734.430m | Latitude | 39°45'45.78893"N | Latitude | 39°45'45.78893"N |
| Easting | 1971088.218m | Longitude | 122°20'14.67896"W | Longitude | 122°20'14.67896"W |
| Elevation | 141.886m | Height | 112.971m | Height | 112.971m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -2164.113m | NS Fwd Azimuth | 239°04'40" |  X | -3825.457m |
|  Easting | -3639.498m | Ell. Distance | 4234.361m |  Y | 746.582m |
|  Elevation | 28.081m |  Height | 27.982m |  Z | -1655.187m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.030 seconds |  X | 0.001m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.001m |
|  Elevation | 0.002m |  Height | 0.002m |  Z | 0.001m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 5.510e-7m2 |  |  |
| Y | 4.676e-7m2 | 1.119e-6m2 |  |
| Z | -4.007e-7m2 | -8.423e-7m2 | 1.376e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | V380 | Y380 |
| Data file: |  | V3801190.rnx | Y3801191.DAT |
| Receiver Type: |  | Z-12 | 4000SSE |
| Receiver Serial Number: |  | 788 | 1547 |
| Antenna type: |  | Ashtech Geodetic III L1/L2 | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Bottom of antenna mount | Bottom of antenna mount |
| Antenna height | Measured | 2.000m | 2.000m |
|  | APC | 2.072m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |