Baseline Summary B1558 (A107 to B107)

| Processed: | Sunday, May 02, 2004 07:39:20AM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 10998.971m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 32.4 |
| Reference variance: | 0.908 |
| RMS: | 0.010m |
| Horizontal Precision 1-sigma (scaled): | 0.001m |
| Vertical Precision 1-sigma (scaled): | 0.002m |

| Start time (GPS Time): | 04/04/28, 19:10:45.000 | 1268, 328245.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/04/28, 20:15:00.000 | 1268, 332100.000 |
| Occupation time: | 01:04:15.000 |  |

Baseline Components (Mark to Mark)

| From: | A107 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 713096.706m | Latitude | 39°35'08.25970"N | Latitude | 39°35'08.25970"N |
| Easting | 1965215.674m | Longitude | 122°24'17.72472"W | Longitude | 122°24'17.72472"W |
| Elevation | 149.996m | Height | 120.861m | Height | 120.861m |

| To: | B107 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 716007.146m | Latitude | 39°36'40.86618"N | Latitude | 39°36'40.86618"N |
| Easting | 1954609.740m | Longitude | 122°31'42.89362"W | Longitude | 122°31'42.89362"W |
| Elevation | 214.395m | Height | 185.406m | Height | 185.406m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 2910.440m | NS Fwd Azimuth | 285°05'25" |  X | -8010.803m |
|  Easting | -10605.934m | Ell. Distance | 10998.519m |  Y | 7195.721m |
|  Elevation | 64.399m |  Height | 64.545m |  Z | 2241.873m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.019 seconds |  X | 0.001m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.001m |
|  Elevation | 0.002m |  Height | 0.002m |  Z | 0.002m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 8.931e-7m2 |  |  |
| Y | 7.705e-7m2 | 1.659e-6m2 |  |
| Z | -6.817e-7m2 | -1.156e-6m2 | 2.493e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | A107 | B107 |
| Data file: |  | A1071193.rnx | B1071193.DAT |
| Receiver Type: |  | Z-12 | 4000SSE |
| Receiver Serial Number: |  | 788 | 1547 |
| Antenna type: |  | Ashtech Geodetic III L1/L2 | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Bottom of antenna mount | Bottom of antenna mount |
| Antenna height | Measured | 2.000m | 2.000m |
|  | APC | 2.072m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |