Baseline Summary B1559 (K852 to P30W)

| Processed: | Sunday, May 02, 2004 07:39:20AM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 6194.291m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 69.9 |
| Reference variance: | 1.731 |
| RMS: | 0.015m |
| Horizontal Precision 1-sigma (scaled): | 0.003m |
| Vertical Precision 1-sigma (scaled): | 0.005m |

| Start time (GPS Time): | 04/04/28, 17:09:45.000 | 1268, 320985.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/04/28, 18:09:15.000 | 1268, 324555.000 |
| Occupation time: | 00:59:30.000 |  |

Baseline Components (Mark to Mark)

| From: | K852 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 725393.386m | Latitude | 39°41'48.91584"N | Latitude | 39°41'48.91583"N |
| Easting | 1983252.554m | Longitude | 122°11'42.95467"W | Longitude | 122°11'42.95467"W |
| Elevation | 71.908m | Height | 43.329m | Height | 43.329m |

| To: | P30W |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 720479.113m | Latitude | 39°39'09.80295"N | Latitude | 39°39'09.80294"N |
| Easting | 1987022.967m | Longitude | 122°09'04.35310"W | Longitude | 122°09'04.35310"W |
| Elevation | 56.255m | Height | 27.755m | Height | 27.755m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -4914.273m | NS Fwd Azimuth | 142°22'48" |  X | 1537.631m |
|  Easting | 3770.412m | Ell. Distance | 6194.237m |  Y | -4654.441m |
|  Elevation | -15.653m |  Height | -15.573m |  Z | -3786.965m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.002m |  NS Fwd Azimuth | 0.032 seconds |  X | 0.002m |
|  Easting | 0.002m |  Ell.Distance | 0.002m |  Y | 0.004m |
|  Elevation | 0.004m |  Height | 0.004m |  Z | 0.002m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 2.634e-6m2 |  |  |
| Y | 3.897e-6m2 | 1.441e-5m2 |  |
| Z | -2.303e-6m2 | -4.890e-6m2 | 3.950e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | K852 | P30W |
| Data file: |  | K8521192.rnx | P30W1192.DAT |
| Receiver Type: |  | Z-12 | 4000SSE |
| Receiver Serial Number: |  | 788 | 1547 |
| Antenna type: |  | Ashtech Geodetic III L1/L2 | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Bottom of antenna mount | Bottom of antenna mount |
| Antenna height | Measured | 2.000m | 2.000m |
|  | APC | 2.072m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |