Baseline Summary B1599 (SUTB to P143)

| Processed: | Wednesday, May 26, 2004 05:02:51PM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 71204.963m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 4.8 |
| Reference variance: | 1.270 |
| RMS: | 0.013m |
| Horizontal Precision 1-sigma (scaled): | 0.001m |
| Vertical Precision 1-sigma (scaled): | 0.004m |

| Start time (GPS Time): | 04/03/23, 15:56:15.000 | 1263, 230175.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/23, 21:00:30.000 | 1263, 248430.000 |
| Occupation time: | 05:04:15.000 |  |

Baseline Components (Mark to Mark)

| From: | SUTB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 670872.383m | Latitude | 39°12'21.02923"N | Latitude | 39°12'21.02923"N |
| Easting | 2015494.934m | Longitude | 121°49'14.13566"W | Longitude | 121°49'14.13566"W |
| Elevation | 646.000m | Height | 617.182m | Height | 617.182m |

| To: | P143 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 741299.773m | Latitude | 39°50'25.19549"N | Latitude | 39°50'25.19548"N |
| Easting | 2005063.958m | Longitude | 121°56'27.01124"W | Longitude | 121°56'27.01124"W |
| Elevation | 58.592m | Height | 31.017m | Height | 31.017m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 70427.391m | NS Fwd Azimuth | 351°41'19" |  X | 15137.297m |
|  Easting | -10430.975m | Ell. Distance | 71199.295m |  Y | 43914.020m |
|  Elevation | -587.408m |  Height | -586.165m |  Z | 53968.211m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.001 seconds |  X | 0.002m |
|  Easting | 0.000m |  Ell.Distance | 0.001m |  Y | 0.003m |
|  Elevation | 0.004m |  Height | 0.004m |  Z | 0.002m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 2.467e-6m2 |  |  |
| Y | 3.732e-6m2 | 6.503e-6m2 |  |
| Z | -3.645e-6m2 | -5.924e-6m2 | 5.982e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | SUTB | P143 |
| Data file: |  | SUTB0830.rnx | P1430831.DAT |
| Receiver Type: |  | Z-12 | 4000SSi |
| Receiver Serial Number: |  | 0802 | 7618 |
| Antenna type: |  | Ashtech Dorne Margolin | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 0.000m | 2.063m |
|  | APC | 0.000m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |