Baseline Summary B1652 (J143 to ORVB)

| Processed: | Wednesday, May 26, 2004 05:02:51PM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Low ratio |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 17045.570m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 2.5 |
| Reference variance: | 0.634 |
| RMS: | 0.009m |
| Horizontal Precision 1-sigma (scaled): | 0.000m |
| Vertical Precision 1-sigma (scaled): | 0.002m |

| Start time (GPS Time): | 04/03/22, 19:56:15.000 | 1263, 158175.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/23, 00:00:00.000 | 1263, 172800.000 |
| Occupation time: | 04:03:45.000 |  |

Baseline Components (Mark to Mark)

| From: | J143 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 716466.387m | Latitude | 39°36'58.48854"N | Latitude | 39°36'58.48853"N |
| Easting | 2027307.081m | Longitude | 121°40'55.13097"W | Longitude | 121°40'55.13097"W |
| Elevation | 62.709m | Height | 35.179m | Height | 35.179m |

| To: | ORVB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 709696.182m | Latitude | 39°33'16.67730"N | Latitude | 39°33'16.67730"N |
| Easting | 2042946.075m | Longitude | 121°30'01.02652"W | Longitude | 121°30'01.02652"W |
| Elevation | 368.001m | Height | 340.718m | Height | 340.718m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -6770.205m | NS Fwd Azimuth | 113°36'29" |  X | 10889.370m |
|  Easting | 15638.993m | Ell. Distance | 17042.335m |  Y | -12090.988m |
|  Elevation | 305.293m |  Height | 305.539m |  Z | -5077.510m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.000m |  NS Fwd Azimuth | 0.005 seconds |  X | 0.001m |
|  Easting | 0.000m |  Ell.Distance | 0.000m |  Y | 0.002m |
|  Elevation | 0.003m |  Height | 0.003m |  Z | 0.002m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 1.260e-6m2 |  |  |
| Y | 1.956e-6m2 | 3.377e-6m2 |  |
| Z | -1.885e-6m2 | -3.071e-6m2 | 3.156e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | J143 | ORVB |
| Data file: |  | J1430822.DAT | ORVB0820.rnx |
| Receiver Type: |  | 4000SSi | Z-12 |
| Receiver Serial Number: |  | 14594 | LP01827 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Ashtech Choke w/Radome |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 1.952m | 0.066m |
|  | APC | 1.952m | 0.066m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |