Baseline Summary B1657 (ORLA to CHO1)

| Processed: | Wednesday, May 26, 2004 05:02:51PM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 58669.933m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 13.2 |
| Reference variance: | 1.697 |
| RMS: | 0.017m |
| Horizontal Precision 1-sigma (scaled): | 0.003m |
| Vertical Precision 1-sigma (scaled): | 0.005m |

| Start time (GPS Time): | 04/03/23, 00:00:00.000 | 1263, 172800.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/23, 01:00:15.000 | 1263, 176415.000 |
| Occupation time: | 01:00:15.000 |  |

Baseline Components (Mark to Mark)

| From: | ORLA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 733339.034m | Latitude | 39°46'06.56750"N | Latitude | 39°46'06.56750"N |
| Easting | 1983520.393m | Longitude | 122°11'32.41833"W | Longitude | 122°11'32.41833"W |
| Elevation | 81.573m | Height | 53.077m | Height | 53.077m |

| To: | CHO1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 696088.298m | Latitude | 39°25'57.51791"N | Latitude | 39°25'57.51791"N |
| Easting | 2028844.009m | Longitude | 121°39'53.84456"W | Longitude | 121°39'53.84456"W |
| Elevation | 45.225m | Height | 17.039m | Height | 17.038m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -37250.736m | NS Fwd Azimuth | 129°17'35" |  X | 25887.779m |
|  Easting | 45323.615m | Ell. Distance | 58669.806m |  Y | -44104.443m |
|  Elevation | -36.348m |  Height | -36.039m |  Z | -28753.817m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.002m |  NS Fwd Azimuth | 0.006 seconds |  X | 0.002m |
|  Easting | 0.001m |  Ell.Distance | 0.002m |  Y | 0.003m |
|  Elevation | 0.004m |  Height | 0.004m |  Z | 0.003m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 2.492e-6m2 |  |  |
| Y | 3.567e-6m2 | 1.224e-5m2 |  |
| Z | -1.169e-6m2 | -3.789e-6m2 | 7.355e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | ORLA | CHO1 |
| Data file: |  | ORLA0821.DAT | CHO10830.rnx |
| Receiver Type: |  | 4000SSE | Z-12 |
| Receiver Serial Number: |  | 1547 | 20101 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Ashtech Geodetic III USCG w/Radome |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 2.063m | 0.000m |
|  | APC | 2.063m | 0.000m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |