Baseline Summary B1664 (ORVB to 1500)

| Processed: | Wednesday, May 26, 2004 05:02:51PM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 37209.943m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 21.7 |
| Reference variance: | 0.722 |
| RMS: | 0.010m |
| Horizontal Precision 1-sigma (scaled): | 0.000m |
| Vertical Precision 1-sigma (scaled): | 0.002m |

| Start time (GPS Time): | 04/03/22, 20:00:15.000 | 1263, 158415.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/23, 00:00:00.000 | 1263, 172800.000 |
| Occupation time: | 03:59:45.000 |  |

Baseline Components (Mark to Mark)

| From: | ORVB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 709696.181m | Latitude | 39°33'16.67728"N | Latitude | 39°33'16.67728"N |
| Easting | 2042946.076m | Longitude | 121°30'01.02647"W | Longitude | 121°30'01.02647"W |
| Elevation | 367.898m | Height | 340.614m | Height | 340.614m |

| To: | 1500 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 705183.061m | Latitude | 39°30'54.08543"N | Latitude | 39°30'54.08543"N |
| Easting | 2006015.391m | Longitude | 121°55'48.16357"W | Longitude | 121°55'48.16357"W |
| Elevation | 29.724m | Height | 1.528m | Height | 1.528m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -4513.120m | NS Fwd Azimuth | 263°20'57" |  X | -32764.412m |
|  Easting | -36930.685m | Ell. Distance | 37207.454m |  Y | 17264.382m |
|  Elevation | -338.173m |  Height | -339.086m |  Z | -3607.531m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.000m |  NS Fwd Azimuth | 0.003 seconds |  X | 0.001m |
|  Easting | 0.000m |  Ell.Distance | 0.000m |  Y | 0.002m |
|  Elevation | 0.003m |  Height | 0.003m |  Z | 0.002m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 1.398e-6m2 |  |  |
| Y | 2.102e-6m2 | 3.534e-6m2 |  |
| Z | -2.043e-6m2 | -3.231e-6m2 | 3.369e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | ORVB | 1500 |
| Data file: |  | ORVB0820.rnx | 15000820.DAT |
| Receiver Type: |  | Z-12 | 4000SSi |
| Receiver Serial Number: |  | LP01827 | 14632 |
| Antenna type: |  | Ashtech Choke w/Radome | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 0.066m | 1.952m |
|  | APC | 0.066m | 1.952m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |