Baseline Summary B730 (1122 to 1500)

| Processed: | Friday, Apr 02, 2004 06:52:59AM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 5671.625m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 53.7 |
| Reference variance: | 0.472 |
| RMS: | 0.007m |
| Horizontal Precision 1-sigma (scaled): | 0.000m |
| Vertical Precision 1-sigma (scaled): | 0.001m |

| Start time (GPS Time): | 04/03/16, 17:43:00.000 | 1262, 236580.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/16, 18:30:15.000 | 1262, 239415.000 |
| Occupation time: | 00:47:15.000 |  |

Baseline Components (Mark to Mark)

| From: | 1122 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 699527.627m | Latitude | 39°27'50.68559"N | Latitude | 39°27'50.68559"N |
| Easting | 2006417.623m | Longitude | 121°55'31.51829"W | Longitude | 121°55'31.51829"W |
| Elevation | 27.184m | Height | -1.200m | Height | -1.200m |

| To: | 1500 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 705184.487m | Latitude | 39°30'54.13170"N | Latitude | 39°30'54.13170"N |
| Easting | 2006013.436m | Longitude | 121°55'48.24537"W | Longitude | 121°55'48.24537"W |
| Elevation | 30.436m | Height | 2.240m | Height | 2.240m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 5656.860m | NS Fwd Azimuth | 355°57'37" |  X | 1562.063m |
|  Easting | -404.187m | Ell. Distance | 5671.623m |  Y | 3262.670m |
|  Elevation | 3.252m |  Height | 3.439m |  Z | 4368.325m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.023 seconds |  X | 0.001m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.001m |
|  Elevation | 0.001m |  Height | 0.001m |  Z | 0.001m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 8.126e-7m2 |  |  |
| Y | 5.929e-7m2 | 1.222e-6m2 |  |
| Z | -4.386e-7m2 | -6.155e-7m2 | 9.957e-7m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | 1122 | 1500 |
| Data file: |  | 11220762-3.DAT | 15000762-3.DAT |
| Receiver Type: |  | 4000SSi | 4000SSi |
| Receiver Serial Number: |  | 6782 | 7613 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 2.063m | 2.063m |
|  | APC | 2.063m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |