Baseline Summary B784 (MINO to C200)

| Processed: | Friday, Apr 02, 2004 06:52:59AM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Precise |
| Met Data: | Standard |
| Baseline slope distance: | 8036.503m |
| Elevation mask: | 15 degrees |
| Variance ratio: | 11.1 |
| Reference variance: | 0.195 |
| RMS: | 0.005m |
| Horizontal Precision 1-sigma (scaled): | 0.000m |
| Vertical Precision 1-sigma (scaled): | 0.001m |

| Start time (GPS Time): | 04/03/16, 15:59:45.000 | 1262, 230385.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/16, 16:45:15.000 | 1262, 233115.000 |
| Occupation time: | 00:45:30.000 |  |

Baseline Components (Mark to Mark)

| From: | MINO |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 699573.946m | Latitude | 39°27'51.98621"N | Latitude | 39°27'51.98621"N |
| Easting | 1988239.166m | Longitude | 122°08'12.01795"W | Longitude | 122°08'12.01795"W |
| Elevation | 31.242m | Height | 2.311m | Height | 2.311m |

| To: | C200 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 693130.030m | Latitude | 39°24'22.74375"N | Latitude | 39°24'22.74374"N |
| Easting | 1983437.844m | Longitude | 122°11'32.31170"W | Longitude | 122°11'32.31170"W |
| Elevation | 29.324m | Height | 0.082m | Height | 0.082m |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | -6443.916m | NS Fwd Azimuth | 216°36'13" |  X | -6236.020m |
|  Easting | -4801.322m | Ell. Distance | 8036.501m |  Y | -918.502m |
|  Elevation | -1.917m |  Height | -2.229m |  Z | -4985.356m |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | 0.014 seconds |  X | 0.001m |
|  Easting | 0.001m |  Ell.Distance | 0.001m |  Y | 0.001m |
|  Elevation | 0.002m |  Height | 0.002m |  Z | 0.002m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 6.683e-7m2 |  |  |
| Y | 9.547e-7m2 | 2.242e-6m2 |  |
| Z | -1.101e-6m2 | -2.311e-6m2 | 3.135e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  | MINO | C200 |
| Data file: |  | MINO0761-3.DAT | C2000761-3.DAT |
| Receiver Type: |  | 4000SSi | 4000SSi |
| Receiver Serial Number: |  | 14594 | 7618 |
| Antenna type: |  | Compact L1/L2 w/Ground Plane | Compact L1/L2 w/Ground Plane |
| Measured To: |  | Antenna Phase Center | Antenna Phase Center |
| Antenna height | Measured | 1.952m | 2.063m |
|  | APC | 1.952m | 2.063m |

Tracking Summary

Residuals

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |