DESIGNATION  - MENDOCINO

PID  - NONE, SET NEW MONUMENT

N 36 30 23.0

# W 119 32 53.1

Set new monument in sidewalk at Mendocino Ave. overpass at Hwy 99.

Additional recovery notes:

Located 23mi southeast of Fresno, 5.5 miles southeast of Selma, and at the south end of Kingsburg, on the Mendocino Ave overpass at Hwy 99.

Located in the pedestrian sidewalk on the west side of the overpass.

21.5 ft west of the C/L of Mendocino Ave.

16.0 ft north of the south end of the pedestrian walk.

5.0 ft west of the east face of the concrete guard rail support

1.6 ft east of the west face of the Bridge Deck.

See photo next page.